# Instruction for Software Upgrade:

- Uninstall the previous revision of the software as the following:

  - Insert the application software CD (3.1.1) into the CD drive

  - While on the command prompt window (Stat-> Programs-> Accessories -> Command Prompt) make the CD drive the current directory

  - From CD drive:> type:    “setup –uninst”

  - Follow the instruction to uninstall the previous software revision (uninstall does not remove you previous data and site specific information)

  - Close the Command Prompt window

- Install the new software revision

  - From “My Computer” open the CD drive and double click the “Setup.exe”

  - Follow the instruction on the screen to complete the installation

- Re-installing the CD writer driver

  - In some cases you may have to re-install the driver for CD writer. To re-install the driver, insert the CD contains the driver and double click on “setup.exe”. follow the instruction on the screen to complete the installation.

- Setting up treadmill and NIBP ports

The new revision of the software communicates with serial ports based on the following assumptions:

Treadmill Port: Com1

NIBP port       : Com2

It may be necessary to set up these ports once the software is installed. To set up port number:

- Right click on “My Computer” and open “Properties”

- From the “system Properties” widow, open the “Hardware” tab and click on “Device Manager”

- In “Device Manager” window, expand the “Ports (Com & LPT)” entry

- Make sure that the com name for treadmill and NIPB are “Com1” and “Com2” correspondingly. You can change the port number (if required) by:

  - Double click on the port entry, select the “Port Setting” tab

  - Click on the Advanced button and change the port number. If the port number is already in use, you should force the change.

Restart the computer once the above steps are completed.